EXP10:      Massive MIMO Capacity vs SNR and Gain

AIM

To simulate and analyze the capacity of a Massive MIMO system with multiple transmit antennas and users, studying the effect of SNR and antenna array gain on system capacity.

THEORY

1. Massive MIMO

- A MIMO system with a large number of transmit antennas (Nt) serving multiple single-antenna users (Nr).

- Provides array gain, spatial multiplexing, and improved spectral efficiency.

2. Capacity of MIMO Channel

The capacity of a MIMO system with Nt transmit and Nr receive antennas:

C=log⁡2det⁡(INr+SNRtotalNtHHH)

Where:

- H= channel matrix (Nr × Nt)

- SNRtotal= linear SNR × Nt (array gain included)

- INr= identity matrix

3. Array Gain

- Increasing Nt enhances received power and capacity due to beamforming gains.

PROCEDURE (Exactly 6 Lines)

- Set the number of users (Nr), transmit antennas (Nt), and SNR range.

- Generate random MIMO channel matrices H for each simulation.

- Compute total SNR including array gain (SNR×Nt).

- Calculate capacity using C=log⁡2det⁡(I+(SNR/Nt)HHH).

- Plot capacity vs SNR for a fixed number of antennas.

- Plot capacity vs number of antennas at fixed SNR to show array gain benefits.

PSEUDO CODE

START

DEFINE Nr, Nt_array, SNR_dB

SET Nt_fixed = 64

FOR each SNR

CONVERT SNR_dB to linear

COMPUTE total SNR including Nt_fixed

GENERATE random MIMO channel H

CALCULATE capacity = log2(det(I + (SNR_total/Nt_fixed) * H*H'))

END FOR

PLOT Capacity vs SNR

SET fixed SNR

FOR each Nt in Nt_array

COMPUTE total SNR including Nt

GENERATE H

CALCULATE capacity = log2(det(I + (SNR_total/Nt) * H*H'))

END FOR

PLOT Capacity vs Number of Antennas

END

Code :

clear; clc; close all;

%% PARAMETERS

N_r = 8;                 % Number of single-antenna users

SNR_dB = 0:5:30;          % SNR per user in dB

N_t_array = [16 32 64 128]; % Number of transmit antennas for gain analysis

%% ------------------ 1. Capacity vs SNR (Fixed N_t) ------------------

N_t_fixed = 64;  % fixed number of antennas

capacity_SNR = zeros(1,length(SNR_dB));

for k = 1:length(SNR_dB)

snr_linear = 10^(SNR_dB(k)/10);       % SNR per user in linear scale

snr_total = snr_linear * N_t_fixed;   % include array gain

% Random MIMO channel

H = (randn(N_r,N_t_fixed) + 1j*randn(N_r,N_t_fixed))/sqrt(2);

% Capacity formula

C = log2(det(eye(N_r) + (snr_total/N_t_fixed)*(H*H')));

capacity_SNR(k) = real(C);

end

% Plot Capacity vs SNR

figure;

plot(SNR_dB, capacity_SNR, '-o','LineWidth',2);

grid on;

xlabel('SNR per User (dB)');

ylabel('Capacity (bits/sec/Hz)');

title(['Massive MIMO Capacity vs SNR (Nt=' num2str(N_t_fixed) ', Nr=' num2str(N_r) ')']);

%% ------------------ 2. Capacity vs Number of Antennas (Gain) ------------------

SNR_fixed_dB = 10;  % fixed SNR per user

SNR_fixed = 10^(SNR_fixed_dB/10);

capacity_gain = zeros(1,length(N_t_array));

for idx = 1:length(N_t_array)

N_t = N_t_array(idx);

snr_total = SNR_fixed * N_t;  % include array gain

H = (randn(N_r,N_t) + 1j*randn(N_r,N_t))/sqrt(2);

C = log2(det(eye(N_r) + (snr_total/N_t)*(H*H')));

capacity_gain(idx) = real(C);

end

% Plot Capacity vs Number of Antennas

figure;

plot(N_t_array, capacity_gain, '-s','LineWidth',2);

grid on;

xlabel('Number of Transmit Antennas (Nt)');

ylabel('Capacity (bits/sec/Hz)');

title(['Massive MIMO Capacity vs Antenna Array Gain (SNR=' num2str(SNR_fixed_dB) ' dB)']);

Output:

Conclusion:

- Capacity increases with SNR, as higher SNR allows more reliable data transmission.

- Capacity also increases with the number of transmit antennas due to array gain and spatial multiplexing.

- Massive MIMO significantly improves spectral efficiency for multi-user systems.

- The simulation demonstrates the benefits of scaling Nt in next-generation wireless networks like 5G and beyond.