EXP 09: Steering Angle Variation in MIMO Beamforming

AIM :

To simulate the effect of varying steering angles in a MIMO beamforming system using MATLAB and analyze how beam direction changes with angle adjustment.

THEORY :

MIMO beamforming uses multiple antennas to transmit signals in a specific direction by adjusting phase shifts across antenna elements.
The direction of the beam is controlled by the steering angle (θ).
For a Uniform Linear Array (ULA), the array factor is:

AF(θ)=n=0N-1ejnkd(sin⁡θ-sin⁡θ0)

Where:

- N= number of antennas

- d= spacing between elements

- k=2πλ(wave number)

- θ0= steering angle

By changing θ0, the beam direction shifts, demonstrating beam steering capability in MIMO systems.

PROCEDURE :

- Define parameters: number of antennas (N), wavelength, element spacing, and angle vector.

- Choose steering angles (e.g., 0°, 20°, 40°, 60°).

- For each steering angle, compute the array factor using MATLAB.

- Normalize the array factor for plotting.

- Plot AF vs. angle to observe beam steering.

- Compare how the main lobe shifts for different angles.

PROGRAM :

% Steering Angle Variation in MIMO Beamforming

clear; clc; close all;

N = 16;                     % Number of antennas

d = 0.5;                    % Spacing (lambda/2)

lambda = 1;                 % Wavelength

theta = -90:0.1:90;         % Angle scan

th = deg2rad(theta);

% Different steering angles to compare

steer_angles = [-30 0 20 45];

colors = {'r','b','g','m'};

figure; hold on;

for s = 1:length(steer_angles)

th0 = deg2rad(steer_angles(s));  % Convert to radians

% Digital beamforming weights

w = exp(1j*2*pi*(d/lambda)*(0:N-1)'*sin(th0));

w = w / sum(abs(w));             % Normalize

% Compute Array Factor

AF = zeros(size(th));

for i = 1:length(th)

ph = exp(1j*2*pi*(d/lambda)*(0:N-1)*sin(th(i)));

AF(i) = abs(sum(w.'.*ph));

end

AF = AF / max(AF);               % Normalize

plot(theta, AF, 'LineWidth', 2, 'Color', colors{s});

end

grid on;

xlabel('Angle (degrees)');

ylabel('Normalized Array Factor');

title('Steering Angle Variation in MIMO Beamforming');

legend('-30°','0°','20°','45°');

OUTPUT :

CONCLUSION :

The MATLAB simulation demonstrates that varying the steering angle effectively changes the direction of the radiated beam in a MIMO beamforming system.
This behavior matches theoretical expectations and illustrates how MIMO beamforming allows directional transmission for improved signal strength, interference reduction, and enhanced system performance.