EXP-15 GFDM vs OFDMA Comparison using MATLAB

AIM: To compare the time-domain waveform characteristics of OFDMA and GFDM modulation schemes by generating and visualizing their signals using MATLAB.

APPARATUS REQUIRED

1.MATLAB / Octave software (with Signal Processing and Plotting tools)

2.Computer or Laptop with minimum 4 GB RAM

3.Simulation script for Array Factor analysis

4.Graph plotting utilities (built into MATLAB)

5.Documentation tools (MS Word for tabulation of results and screenshots

THEORY:

# OFDMA (Orthogonal Frequency Division Multiple Access)

# Uses orthogonal subcarriers.

# Each subcarrier carries one symbol without pulse shaping.

# Simple implementation but produces high out-of-band emissions (OOBE).

# Suitable for LTE and 5G sub-6 GHz.

# GFDM (Generalized Frequency Division Multiplexing)

# Multi-carrier scheme with subcarriers and subsymbols arranged in a block.

# Uses pulse shaping (e.g., raised cosine) to reduce interference.

# Offers low OOBE and flexibility but is more complex.

# Considered for 5G waveforms.

# Key difference:

# OFDMA → purely orthogonal, rectangular pulses.

# GFDM → non-orthogonal, pulse-shaped symbols.

# PROCEDURE

- Define number of subcarriers, subsymbols, modulation order, and samples per symbol.

- Generate random QPSK symbols for each subcarrier and subsymbol.

- OFDMA generation:

- Upsample each symbol.

- Modulate each onto a cosine at the subcarrier index.

- Add signals across subcarriers.

- GFDM generation:

- Upsample data.

- Apply raised cosine pulse shaping.

- Modulate onto subcarriers.

- Plot:

- QPSK symbol values

- OFDMA waveform

- GFDM waveform

CODE:

clear; clc; close all;

%% PARAMETERS

N_subcarriers = 8;     % Number of subcarriers

N_subsymbols = 4;      % Number of subsymbols per subcarrier

M = 4;                 % QPSK modulation

sps = 8;               % Samples per symbol for pulse shaping

slice_names = {'OFDMA','GFDM'}; % For legend

%% GENERATE RANDOM DATA

data = randi([0 M-1], N_subcarriers, N_subsymbols);

% QPSK mapping: 0->1+1j, 1->1-1j, 2->-1+1j, 3->-1-1j

data_mod = exp(1j*2*pi*data/M);

%% -------------------- OFDMA WAVEFORM --------------------

ofdm_signal = zeros(1, N_subcarriers*N_subsymbols*sps);

for k = 1:N_subcarriers

sym_up = upsample(real(data_mod(k,:)), sps); % upsample for time resolution

t_index = (1:length(sym_up)) + (k-1)*length(sym_up);

ofdm_signal(t_index) = ofdm_signal(t_index) + sym_up .* cos(2*pi*(k-1)/N_subcarriers*(0:length(sym_up)-1));

end

%% -------------------- GFDM WAVEFORM --------------------

beta = 0.25;                  % Roll-off factor

span = 4;                      % Filter span in symbols

rc_filter = rcosdesign(beta, span, sps); % Raised Cosine filter

gfdm_signal = zeros(1, N_subcarriers*N_subsymbols*sps);

for k = 1:N_subcarriers

sym_up = upsample(real(data_mod(k,:)), sps);      % upsample

filt_sym = conv(sym_up, rc_filter, 'same');      % apply pulse shaping

t_index = (1:length(filt_sym)) + (k-1)*length(filt_sym);

gfdm_signal(t_index) = gfdm_signal(t_index) + filt_sym .* cos(2*pi*(k-1)/N_subcarriers*(0:length(filt_sym)-1));

end

%% -------------------- PLOTS --------------------

figure;

% a. Random QPSK data

subplot(3,1,1);

stem(real(data_mod(:)),'filled');

title('Random QPSK Data for Subcarriers and Subsymbols');

xlabel('Symbol Index'); ylabel('Amplitude'); grid on;

% b. OFDMA waveform

subplot(3,1,2);

plot(ofdm_signal,'LineWidth',1.5);

title('OFDMA Waveform (Time Domain)');

xlabel('Sample Index'); ylabel('Amplitude'); grid on;

% c. GFDM waveform

subplot(3,1,3);

plot(gfdm_signal,'LineWidth',1.5);

title('GFDM Waveform with Pulse Shaping (Time Domain)');

xlabel('Sample Index'); ylabel('Amplitude'); grid on;

OUTPUTS:

RESULTS:

The MATLAB comparison confirms that OFDMA provides a simple, orthogonal signal structure, while GFDM offers improved spectral characteristics through pulse shaping. Although GFDM yields smoother time-domain signals with lower out-of-band emissions, it introduces implementation complexity. Therefore, OFDMA is preferred for legacy LTE systems, while GFDM provides flexibility for advanced 5G physical layer designs.