Exp 16:          mmWave Loss Components (28 GHz & 60 GHz)

AIM

To simulate and compare the Reflection Loss, Penetration Loss, and Atmospheric Loss of mmWave signals at 28 GHz and 60 GHz over varying distances using MATLAB.

THEORY (Short & Clear)

1. mmWave Communication

- Operates in high-frequency bands (24–100 GHz).

- Used in 5G due to high bandwidth but suffers higher propagation losses.

2. Reflection Loss

- Calculated using Fresnel reflection coefficient.

- Depends on incident angle and wall permittivity.

3. Penetration Loss

- Higher frequencies penetrate walls poorly → more loss at 60 GHz.

4. Atmospheric Absorption

- O₂ molecules cause strong attenuation at 60 GHz (~0.2 dB/m).

- 28 GHz has lower atmospheric attenuation (~0.06 dB/m

PROCEDURE (Exactly 6 Lines)

- Define frequency, distance range, wall permittivity, and atmospheric loss parameters.

- Compute Fresnel reflection coefficient for each frequency and convert it to dB loss.

- Estimate penetration loss using a frequency-dependent model.

- Compute atmospheric loss by multiplying attenuation (dB/m) with distance.

- Store all three loss components for 28 GHz and 60 GHz.

- Plot reflection, penetration, and atmospheric losses against distance for comparison.

PSEUDO CODE:

START

DEFINE frequencies = [28 GHz, 60 GHz]

DEFINE distances = 1 to 100 meters

DEFINE permittivity and incident angle

DEFINE atmospheric attenuation values

FOR each frequency

FOR each distance

Compute Fresnel reflection coefficient → ReflectionLoss

Compute frequency-based penetration loss → PenetrationLoss

Compute AtmosphericLoss = attenuation × distance

END FOR

END FOR

PLOT Reflection, Penetration, and Atmospheric Losses vs Distance

END

CODE:

clear; clc; close all;

%% PARAMETERS

frequencies = [28e9 60e9];        % Frequencies in Hz

c = 3e8;                          % Speed of light (m/s)

distances = 1:1:100;              % Distance in meters

epsilon_r_wall = 5;               % Relative permittivity of wall

theta_inc = 45;                    % Incident angle in degrees

% Atmospheric attenuation (dB/m)

atm_loss = [0.06, 0.2];           % 28GHz, 60GHz

%% INITIALIZE ARRAYS

ReflectionLoss = zeros(length(frequencies), length(distances));

PenetrationLoss = zeros(length(frequencies), length(distances));

AtmosphericLoss = zeros(length(frequencies), length(distances));

%% COMPUTE LOS / LOS COMPONENTS

for f_idx = 1:length(frequencies)

f = frequencies(f_idx);

for d_idx = 1:length(distances)

d = distances(d_idx);

% -------- Reflection Loss (Fresnel coefficient, perpendicular polarization) --------

theta_rad = deg2rad(theta_inc);

gamma = (cos(theta_rad) - sqrt(epsilon_r_wall - sin(theta_rad)^2)) / ...

(cos(theta_rad) + sqrt(epsilon_r_wall - sin(theta_rad)^2));

ReflectionLoss(f_idx,d_idx) = abs(gamma)^2;             % Power reflection coefficient

ReflectionLoss(f_idx,d_idx) = -10*log10(ReflectionLoss(f_idx,d_idx)+eps); % dB

% -------- Penetration Loss (frequency-dependent) --------

PenetrationLoss(f_idx,d_idx) = 5 + 0.02*f/1e9;         % Example: 28GHz ~5.56dB, 60GHz ~6.2dB

% -------- Atmospheric Loss (linear with distance) --------

AtmosphericLoss(f_idx,d_idx) = atm_loss(f_idx) * d;     % dB

end

end

%% -------------------- PLOTS --------------------

figure;

% Reflection Loss

plot(distances, ReflectionLoss(1,:),'r','LineWidth',1.5); hold on;

plot(distances, ReflectionLoss(2,:),'b','LineWidth',1.5);

% Penetration Loss

plot(distances, PenetrationLoss(1,:),'r--','LineWidth',1.5);

plot(distances, PenetrationLoss(2,:),'b--','LineWidth',1.5);

% Atmospheric Loss

plot(distances, AtmosphericLoss(1,:),'r-.','LineWidth',1.5);

plot(distances, AtmosphericLoss(2,:),'b-.','LineWidth',1.5);

grid on;

xlabel('Distance (m)'); ylabel('Loss (dB)');

title('mmWave Loss Components at 28 GHz and 60 GHz');

legend('Reflection 28 GHz','Reflection 60 GHz', ...

'Penetration 28 GHz','Penetration 60 GHz', ...

'Atmospheric 28 GHz','Atmospheric 60 GHz');

Output:

Conclusion:

  60 GHz experiences significantly higher losses than 28 GHz due to higher penetration loss and strong atmospheric absorption.

  Reflection loss remains similar over distance but depends strongly on wall material and frequency.

  Atmospheric absorption increases linearly with distance, dominating total loss at 60 GHz.

  The results justify why 28 GHz is favored for longer-range 5G links, while 60 GHz is suitable mainly for short-range indoor or hotspot communication.