EXP 18   5G NR Frame Structure Analysis with different Subcarrier Spacing

AIM

To simulate 5G network slicing by dynamically allocating bandwidth to eMBB, URLLC, and mMTC slices based on priority and evaluating their throughput, latency, and bandwidth distribution.

THEORY

1. Network Slicing

- A 5G technique that divides a physical network into multiple logical slices, each optimized for a different service.

- Each slice has unique QoS parameters such as data rate, latency, bandwidth, and reliability.

2. Three Major 5G Slices

- eMBB (Enhanced Mobile Broadband): High throughput, large bandwidth.

- URLLC (Ultra-Reliable Low-Latency Communications): High priority, low latency, mission-critical services.

- mMTC (Massive Machine-Type Communications): Handles many devices, low data-rate IoT traffic.

3. Priority-Based Allocation

- Bandwidth is dynamically distributed using weighted priorities:

BWalloc=BWtotal×Pi∑P

4. Throughput

T=BW×Spectral Efficiency

5. Latency

- URLLC must maintain sub-millisecond latency.

- eMBB and mMTC allow higher latency.

PROCEDURE (Exactly 6 Lines)

- Define total bandwidth and the bandwidth/throughput requirements for eMBB, URLLC, and mMTC.

- Assign priority values to each slice based on QoS needs.

- Compute dynamic bandwidth allocation using normalized priority weights.

- Calculate throughput using allocated bandwidth and spectral efficiency.

- Simulate random latency values representing realistic 5G slice performance.

- Plot throughput, latency, and bandwidth allocation for all slices.

PSEUDO CODE

START

SET total bandwidth and slice requirements

DEFINE priority weights for eMBB, URLLC, mMTC

CALCULATE BW_alloc = total_BW * (priority / sum(priority))

COMPUTE throughput = BW_alloc * spectral_efficiency

SIMULATE latency values for each slice

DISPLAY results and PLOT throughput, latency, and BW allocation

END

CODE:

clear; clc; close all;

%% Total available bandwidth (MHz)

BW_total = 100;

%% Slice Requirements

% eMBB: High throughput

% URLLC: Ultra low latency, high priority

% mMTC: Large number of devices, small packets

req_eMBB = 50;   % Required Mbps

req_URLLC = 20;  % Required Mbps

req_mMTC = 10;   % Required Mbps

priority = [2 3 1];  % 1=low, 3=high priority (URLLC > eMBB > mMTC)

%% Dynamic Bandwidth Allocation

BW_alloc = zeros(1,3);

req = [req_eMBB req_URLLC req_mMTC];

% Weighted allocation based on priority

priority_weights = priority ./ sum(priority);

BW_alloc = BW_total * priority_weights;

%% Compute Throughput (T = BW * spectral efficiency)

SE_eMBB = 6;    % bits/s/Hz

SE_URLLC = 4;

SE_mMTC = 1;

th_eMBB = BW_alloc(1)*1e6 * SE_eMBB / 1e6;

th_URLLC = BW_alloc(2)*1e6 * SE_URLLC / 1e6;

th_mMTC = BW_alloc(3)*1e6 * SE_mMTC / 1e6;

%% Latency Simulation (URLLC minimum)

lat_eMBB = 20 + rand*10;    % ms

lat_URLLC = 1 + rand*0.5;   % ms

lat_mMTC = 50 + rand*20;    % ms

%% Print Results

fprintf("\n===== Network Slicing Simulation Results =====\n");

fprintf("Allocated Bandwidth (MHz): eMBB=%.2f, URLLC=%.2f, mMTC=%.2f\n", BW_alloc(1), BW_alloc(2), BW_alloc(3));

fprintf("Throughput (Mbps): eMBB=%.2f, URLLC=%.2f, mMTC=%.2f\n", th_eMBB, th_URLLC, th_mMTC);

fprintf("Latency (ms): eMBB=%.2f, URLLC=%.2f, mMTC=%.2f\n", lat_eMBB, lat_URLLC, lat_mMTC);

%% Plot Throughput Comparison

figure;

bar([th_eMBB, th_URLLC, th_mMTC]);

set(gca, 'XTickLabel', {'eMBB','URLLC','mMTC'});

ylabel('Throughput (Mbps)');

title('Slice-wise Throughput Performance');

%% Plot Latency Comparison

figure;

bar([lat_eMBB, lat_URLLC, lat_mMTC]);

set(gca, 'XTickLabel', {'eMBB','URLLC','mMTC'});

ylabel('Latency (ms)');

title('Slice-wise Latency Performance');

%% Plot Bandwidth Allocation

figure;

pie(BW_alloc, {'eMBB','URLLC','mMTC'});

title('Dynamic Bandwidth Allocation for 3 Network Slices');

OUTPUT

Conclusion :

  Priority-based dynamic slicing allocates more bandwidth to URLLC, ensuring low latency and reliability.

  eMBB achieves the highest throughput as expected due to higher spectral efficiency.

  mMTC receives minimal bandwidth but still supports a large number of small IoT packets.

  The simulation demonstrates how 5G slicing efficiently balances speed, reliability, and scalability among different services.