EXP 5 MIMO System Simulation with BER Analysis

Objective:

- To implement 2×2 and 4×4 MIMO to study spatial multiplexing and improve link reliability.

- To analyze throughput, spectral efficiency, and BER to evaluate overall downlink performance.

Theory:

### 1. LTE (Long Term Evolution)

It a 4G wireless communication standard that provides high-speed data transmission using advanced digital modulation and coding techniques.

### 2. OFDMA (Orthogonal Frequency Division Multiple Access)

It divides the bandwidth into many orthogonal subcarriers, allowing simultaneous data transmission to multiple users with reduced interference

### 3. MIMO (Multiple Input Multiple Output)

MIMO uses multiple transmit and receive antennas to increase data rates through spatial multiplexing and improve reliability through spatial diversity.

### 4. LTE Downlink

The LTE downlink combines OFDMA and MIMO to achieve high spectral efficiency, robust performance, and enhanced throughput in real-world wireless environments.

### 5. Effect of parameter metrics

Performance metrics such as Bit Error Rate (BER), throughput, and spectral efficiency help evaluate the effectiveness and reliability of LTE downlink communication systems.

Procedure:

- Define system parameters: number of subcarriers (Nsubcarriers), modulation order   (modOrder), MIMO streams (MIMOStreams).

- Calculate bits per symbol and throughput per user.

3.  Generate random bitstream for one user.

4. Map bits to 16-QAM symbols.

5. Assign symbols to subcarriers and generate OFDM signal using IFFT.

6. Plot the OFDM signal spectrum using FFT.

7. Plot the time-domain OFDM signal (real part).

8. Plot the 16-QAM constellation diagram for transmitted symbols.

9. Display calculated parameters: number of subcarriers, throughput per user, total throughput with MIMO.

Code:

clear; clc; close all;

%% PARAMETERS

numBits = 1e5; SNRdB = 0:2:20; bps = 2;

%% STUDENT INPUT: CHANGE MIMO ORDER

Nt = input('Transmit antennas Nt: ');

Nr = input('Receive antennas Nr: ');

%% QPSK SYMBOLS

bits = randi([0 1], numBits,1);

bits_reshape = reshape(bits, bps, []).';

symbols = (2*bits_reshape(:,1)-1) + 1j*(2*bits_reshape(:,2)-1);

symbols = symbols/sqrt(2);

numSym = length(symbols);

%% SIMULATION

BER = zeros(1,length(SNRdB));

for s = 1:length(SNRdB)

snr = 10^(SNRdB(s)/10);

noiseVar = 1/snr;

% Reshape symbols for MIMO

sym_tx = reshape(symbols, Nt, []).';

% Channel & noise

H = (randn(Nr,Nt,size(sym_tx,1))+1j*randn(Nr,Nt,size(sym_tx,1)))/sqrt(2);

n = sqrt(noiseVar/2)*(randn(Nr,size(H,3))+1j*randn(Nr,size(H,3)));

% Receive & Zero-Forcing

y = zeros(Nr,size(H,3));

x_hat = zeros(Nt,size(H,3));

for k=1:size(H,3)

y(:,k) = H(:,:,k)*sym_tx(k,:).' + n(:,k);

x_hat(:,k) = pinv(H(:,:,k))*y(:,k);

end

% Demodulate

x_hat_vec = x_hat.'; x_hat_vec = x_hat_vec(:);

bits_rx = zeros(numBits,1);

bits_rx(1:2:end) = real(x_hat_vec)>0; bits_rx(2:2:end) = imag(x_hat_vec)>0;

BER(s) = sum(bits~=bits_rx)/numBits;

end

%% PLOT

figure; semilogy(SNRdB, BER,'-o','LineWidth',2);

grid on; xlabel('SNR (dB)'); ylabel('BER');

title(sprintf('%dx%d MIMO BER vs SNR (QPSK, ZF)', Nt, Nr));

PSEUDO CODE :

BEGIN

1. Initialize system parameters:

Set Nt, Nr, modulation order M, detector type (ZF/MMSE)

Set OFDM parameters: Nfft, Nused, CP length, subcarrier spacing

Compute sampling frequency and OFDM symbol duration

Compute bitsPerSymbol = log2(M)

Set number of OFDM symbols and SNR range

Set number of spatial streams = Nt

2. Initialize result arrays:

BER[], Throughput[], SpectEff[]

3. FOR each SNR value:

Set noise variance based on SNR

Initialize totalErrorBits = 0, totalBits = 0, totalSuccessBits = 0

FOR each OFDM symbol:

a. TRANSMITTER:

FOR each transmit stream:

Generate random bits

Map bits → QAM symbols

Store symbols in txSymbols(stream, subcarriers)

b. CHANNEL (per subcarrier):

FOR each subcarrier:

Generate Rayleigh channel matrix H

Extract transmitted symbol vector s

Generate noise vector n

Compute received vector y = H*s + n

Store y and H

c. RECEIVER DETECTION:

FOR each subcarrier:

Retrieve H and y

IF detector = ZF:

Compute W = pseudo-inverse(H)

ELSE (MMSE):

Compute W = (H'H + noiseVar*I)^(-1) * H'

Estimate symbols s_hat = W * y

Store s_hat

d. DEMODULATION AND ERROR COUNT:

Convert all estimated symbols → bits

Compare with transmitted bits

Count error bits

Update totalErrorBits, totalBits, totalSuccessBits

END FOR OFDM symbols

Compute BER = totalErrorBits / totalBits

Compute avgSuccessBits = totalSuccessBits / numOFDMsymbols

Compute Throughput = avgSuccessBits / OFDMsymbolDuration

Compute SpectEff = Throughput / bandwidth

Store results in BER[], Throughput[], SpectEff[]

END FOR SNR values

4. Plot:

BER vs SNR

Throughput vs SNR

Spectral Efficiency vs SNR

5. Display summary table of all metrics

OUTPUT:

4 x 4 MIMO

2 x 2 MIMO

FLOW CHART :

CONSOLE OUTPUT:

4 x 4 MIMO

2 x 2 MIMO

Conclusion:

- The throughput per user and total throughput increased with the number of MIMO streams.

- The OFDM spectrum, time-domain signal, and 16-QAM constellation provided insights into the LTE physical layer.

- This minimal simulation can be extended to multiple users, higher-order MIMO, and larger OFDM symbols for more realistic LTE system analysis.