EXP 12: 5G Throughput vs SNR with Modulation Labels

AIM :

To analyze and compare the throughput performance of different 5G modulation schemes (QPSK, 16QAM, 64QAM) as a function of Signal-to-Noise Ratio (SNR) using Shannon capacity theory, and to visualize their throughput curves on a single plot.

THEORY :

### 5G Modulation Schemes

5G uses adaptive modulation to efficiently utilize spectrum depending on channel quality:

- QPSK → 2 bits/symbol

- 16QAM → 4 bits/symbol

- 64QAM → 6 bits/symbol

Higher-order modulation increases data rate but requires higher SNR.

### 2. Signal-to-Noise Ratio (SNR)

SNR (in dB) is a measure of signal strength relative to background noise:

SNRlinear=10SNRdB10

Higher SNR → better channel quality → supports higher modulation.

### 3. Shannon Capacity

The theoretical maximum data rate (capacity) of a channel is:

C=Blog⁡2(1+SNR)

where
B= bandwidth (100 MHz),
SNR= linear SNR.

This capacity is used as a reference to estimate throughput for each modulation.

### 4. Throughput Scaling

Throughput is scaled relative to bits per symbol:

Throughput=C×k6

where
k= bits per symbol for the modulation scheme,
6 = bits per symbol for 64-QAM (reference).

This models practical 5G performance, where higher modulations approach Shannon capacity more closely.

PROCEDURE :

- Initialize Parameters

  - Bandwidth = 100 MHz

  - SNR range = 0 to 30 dB

  - Modulation schemes = QPSK, 16QAM, 64QAM

  - Bits per symbol = 2, 4, 6

- Create an array to store throughput values for each modulation and SNR level.

- Convert SNR from dB to linear using

10SNRdB10

- Compute Shannon capacity for each SNR value.

- Scale throughput using bit-per-symbol efficiency relative to 64-QAM.

- Store the throughput values in a matrix.

- Plot throughput vs SNR

  - Use different colors for each modulation

  - Add labels at end of curves

  - Mark axes, title, and grid

- Analyze the plot comparing throughput of each modulation scheme.

PROGRAM :

clear; clc; close all;

%% PARAMETERS

BW = 100e6;                          % Bandwidth in Hz (100 MHz)

SNR_dB = 0:5:30;                     % SNR range in dB

mod_schemes = {'QPSK','16QAM','64QAM'};  % Modulation schemes

bits_per_symbol = [2 4 6];               % Bits per symbol

throughput = zeros(length(mod_schemes), length(SNR_dB));

%% CALCULATE THROUGHPUT

for m = 1:length(mod_schemes)

k = bits_per_symbol(m);  % bits per symbol

for s = 1:length(SNR_dB)

snr_linear = 10^(SNR_dB(s)/10);  % Convert dB to linear

C = BW * log2(1 + snr_linear);    % Shannon capacity in bits/sec

throughput(m,s) = C * (k/6);      % Adjust relative to 64QAM efficiency

end

end

%% PLOT THROUGHPUT

figure;

colors = {'b','r','g'}; hold on;

for m = 1:length(mod_schemes)

plot(SNR_dB, throughput(m,:)/1e6, 'LineWidth',2, 'Color', colors{m});

% Add text label at the end of the curve

text(SNR_dB(end)+0.5, throughput(m,end)/1e6, mod_schemes{m}, 'Color', colors{m}, ...

'FontWeight','bold','FontSize',10);

end

grid on;

xlabel('SNR (dB)');

ylabel('Throughput (Mbps)');

title('5G Throughput vs SNR for Different Modulation Schemes');

xlim([min(SNR_dB) max(SNR_dB)+5]);  % leave space for label

OUTPUT :

CONCLUSION :

The throughput for all modulation schemes increases as SNR increases.Higher-order modulations like 64QAM provide higher throughput but require high SNR to perform efficiently.
Thus, 5G adapts modulation based on channel quality to achieve the best possible data rate.