11. Spectrum Analysis of Sub-1 GHz, Mid-band, and mmWave by Plotting Propagation Loss at 700 MHz, 3.5 GHz, and 28 GHz

AIM:

To analyze and compare the path loss characteristics of three major 5G spectrum bands—700 MHz (Sub-1 GHz), 3.5 GHz (Mid-band), and 28 GHz (mmWave)—over distances up to 5 km using COST-231 Hata and Free-Space path loss models.

TOOLS REQUIRED:

- MATLAB

- Communications Toolbox

- Signal Processing Toolbox

- Laptop/PC

THEORY:

Wireless signal attenuation increases with distance and frequency.

- Sub-1 GHz (700 MHz) experiences lower path loss and supports long-range coverage.

- Mid-band (3.5 GHz) provides a balance between range and capacity.

- mmWave (28 GHz) suffers high path loss but enables extremely high data rates at short distances.

Propagation models used:

- COST-231 Hata model → applicable for Sub-6 GHz urban environments.

- Free Space Path Loss (FSPL) → used for mmWave bands with an added 2 dB atmospheric absorption loss.

PROCEDURE:

- Define operating bands: 700 MHz, 3.5 GHz, and 28 GHz.

- Set distance range from 10 m to 5 km.

- Calculate path loss for Sub-6 GHz bands using the COST-231 Hata model.

- Compute mmWave path loss using FSPL with atmospheric attenuation.

- Plot path loss vs distance for all three bands on a single graph.

- Label each curve for easy comparison.

CODE:

clear; clc; close all;

%% PARAMETERS

c = 3e8;                         % Speed of light (m/s)

d = 10:10:5000;                  % Distance in meters

freq = [0.7e9, 3.5e9, 28e9];     % Frequencies: 700 MHz, 3.5 GHz, 28 GHz

band_names = {'700 MHz (Sub-1 GHz)','3.5 GHz (Mid-band)','28 GHz (mmWave)'};

PL = zeros(length(freq), length(d));

%% BASE STATION & MOBILE HEIGHT

h_bs = 30;  % Base station height (m)

h_ms = 1.5; % Mobile station height (m)

%% PATH LOSS CALCULATION

for i = 1:length(freq)

f_MHz = freq(i)/1e6;

if freq(i) <= 6e9   % Sub-6 GHz → COST-231 Hata

if f_MHz <= 200

a_hms = 8.29*(log10(1.54*h_ms))^2 - 1.1;

else

a_hms = 3.2*(log10(11.75*h_ms))^2 - 4.97;

end

PL(i,:) = 46.3 + 33.9*log10(f_MHz) - 13.82*log10(h_bs) - a_hms + ...

(44.9 - 6.55*log10(h_bs))*log10(d/1000);

else                % mmWave → Free space path loss

lambda = c/freq(i);

PL(i,:) = 20*log10(4*pi*d/lambda) + 2;  % FSPL + 2 dB atmospheric loss

end

end

%% PLOT

figure;

colors = {'b','r','g'};

hold on;

for i = 1:length(freq)

plot(d, PL(i,:), 'LineWidth', 2, 'Color', colors{i});

% Add text label near the end of the line

text(d(end-10), PL(i,end-10), band_names{i}, 'Color', colors{i}, 'FontWeight','bold');

end

grid on;

xlabel('Distance (m)');

ylabel('Path Loss (dB)');

title('Path Loss vs Distance for Different Frequency Bands');

OUTPUT:

RESULT:

This experiment confirms that lower frequencies support long-distance communication with less path loss, while higher frequencies such as mmWave suffer significant propagation loss and require dense small-cell deployments. Thus, Sub-1 GHz, Mid-band, and mmWave bands each play a unique and complementary role in 5G communication networks.