EXP- 13 NB-IoT vs LTE-M Throughput Simulation

AIM: To compare the throughput performance of NB-IoT and LTE-M technologies by analyzing how throughput varies with SNR and bandwidth using Shannon capacity–based simulation.

APPARATUS REQUIRED

1.MATLAB / Octave software (with Signal Processing and Plotting tools)

2.Computer or Laptop with minimum 4 GB RAM

3.Simulation script for Array Factor analysis

4.Graph plotting utilities (built into MATLAB)

5.Documentation tools (MS Word for tabulation of results and screenshots

THEORY:

# NB-IoT and LTE-M are low-power wide-area (LPWA) technologies used in 4G/5G IoT networks.

# NB-IoT

# Bandwidth: 180 kHz

# High coverage, low power

# Higher latency → lower throughput

# LTE-M

# Bandwidth: 1.4 MHz

# Lower latency

# Supports higher throughput and mobility

# Using Shannon capacity:

# C=Blog⁡2(1+SNR)

# Latency is included by scaling the effective throughput:

# T=Clatency
PROCEDURE

- Define bandwidth and latency for NB-IoT and LTE-M.

- Choose an SNR range from 0 to 30 dB.

- Compute throughput vs SNR for each technology using the Shannon formula.

- Plot throughput (in kbps) versus SNR for both systems.

- Fix SNR = 20 dB and calculate throughput vs bandwidth.

- Plot a bar graph comparing NB-IoT and LTE-M throughput at 20 dB SNR.

CODE:

clear; clc; close all;

%% PARAMETERS

BW = [180e3, 1.4e6];           % Bandwidth: NB-IoT, LTE-M (Hz)

latency = [1, 0.1];             % Latency in seconds (NB-IoT, LTE-M)

SNR_dB = 0:5:30;                % SNR range in dB

tech = {'NB-IoT','LTE-M'};      % Technology names

%% Throughput vs SNR

throughput = zeros(length(BW), length(SNR_dB));

for t = 1:length(BW)

for s = 1:length(SNR_dB)

snr_linear = 10^(SNR_dB(s)/10);

C = BW(t) * log2(1 + snr_linear);          % Shannon capacity

throughput(t,s) = C * (1/latency(t));      % Adjust for latency

end

end

% Plot Throughput vs SNR

figure; hold on;

colors = {'b','r'};

for t = 1:length(BW)

plot(SNR_dB, throughput(t,:)/1e3, 'LineWidth',2,'Color',colors{t}); % kbps

text(SNR_dB(end)+0.5, throughput(t,end)/1e3, tech{t}, 'Color',colors{t}, ...

'FontWeight','bold','FontSize',10);

end

grid on;

xlabel('SNR (dB)');

ylabel('Throughput (kbps)');

title('NB-IoT vs LTE-M Throughput vs SNR');

%% Throughput vs Bandwidth at Fixed SNR

fixed_SNR_dB = 20;  % Example SNR

throughput_bw = zeros(length(BW),1);

for t = 1:length(BW)

snr_linear = 10^(fixed_SNR_dB/10);

C = BW(t) * log2(1 + snr_linear);

throughput_bw(t) = C * (1/latency(t));

end

% Plot Throughput vs Bandwidth

figure;

bar(throughput_bw/1e3);  % kbps

set(gca,'XTickLabel',tech,'FontWeight','bold');

ylabel('Throughput (kbps)');

title(['Throughput vs Bandwidth at SNR = ' num2str(fixed_SNR_dB) ' dB']);

grid on;

OUTPUTS:

RESULTS:

The analysis confirms that NB-IoT is optimized for low-data-rate, long-coverage IoT applications, while LTE-M supports higher throughput and mobility. Throughput improves with SNR for both systems, but LTE-M outperforms NB-IoT due to its wider bandwidth and faster latency characteristics.